[image: image3.jpg]




[image: image1.jpg]



[image: image2.jpg]
































































































































































































































































































�





OWN'S Automatic Toothbrush Manufacturing Machines


牙刷專業機械整廠設備-塑膠處理機具





VERTICAL PLASTIC INJECTION MOLDING MACHINE立式塑膠射出成型機





HORIZNTAL PLASTIC INJECTION MOLDING MACHINE臥式塑膠射出成型機














COLOR MIXING M-


ACHINE OF PLAST-


IC MATERIAL


塑膠原料色粉混合機





DESCRIPTION�
說明�
UNIT�
HORIZNTAL TYPE�
VERTICAL TYPE�
�
INJECTION�
射出部份�
�
臥式125T�
立式55T�
�
Screw Diameter�
螺旋桿直徑�
mm�
45�
32�
�
Theoretical in jection Volume�
理論射出容積�
cm³�
389�
90�
�
In jection Pressure�
射膠壓力�
kg/cm²�
1839�
1030�
�
In jection Rate�
射出率�
cm³/sec�
120�
104�
�
Shot Weight�
射膠量�
grams�
326�
82�
�
CLAMPING UNIT�
夾模部份�
�
Distance Between Tie Bars(H×V)�
夾模最大尺寸�
mm�
420×420�
400×250�
�
Mold Platen�
盤面尺寸�
mm�
620×620�
580×430�
�
Mold Height�
模具之厚薄�
mm�
100-450�
150-280�
�
Die Opening Stroke�
開模行程�
mm�
400�
200�
�
Diameter of Centering Ring�
前機壁模環直徑�
mm�
120�
90�
�
Ejector Stroke�
托模行程�
mm�
110�
45/95�
�
Ciamping Force�
鎖模力�
tons�
125�
45�
�
GENERAL DATA�
一般資料�
�
Number of Heating Zones�
電熱段數�
zones�
4�
1+2�
�
Heating Capacity�
電熱器容量�
kw�
7.6�
3.5�
�
Pump Driving Motor�
馬達出力�
hp�
25�
7.5�
�
Machine Size(L×W×H)�
機械尺寸�
m�
5.4×1.4×1.5�
2.6×1.2×3.1�
�
Net Weight�
機械淨重�
kgs�
4000�
2600�
�
Packing Size (L×W×H)�
機械裝箱後尺寸�
m�
5.6×1.8×2.2�
2.7×1.3×2.3�
�
Shipping Weight�
裝箱後總重�
kgs�
4500�
2900�
�















Explanation:


It is used to inject and stuff two-color toothbrush handles with stopping slide soft plastic.It can inject and stuff 8pcs of toothbrush handles per about 30~40secounds per shot.





說明：


其一次可再三十至四十秒內射出八至十二支單種原料之牙刷柄。(視牙刷而定，一般壹模皆８支)








Explanation:�It is used to inject 8~12pcs (depends on what kind of toothbrush, usually it's 8 cavities per mold) of single color toothbrush handles per 30~40 second per shot.





說明：�用於製造第二色牙刷柄，對握把的防滑軟膠部分進行射出充填作業﹔可在三十至四十秒內充填八支刷柄。



























































PAGE  
1

